# InBody Clinical Question Bank – 40+ High-Precision FAQs

## 1. Core Technology & Validity

Q1. What core technology does InBody use for body composition analysis?
A. Direct Segmental Multi-Frequency Bioelectrical Impedance Analysis (DSM-BIA).

Q2. How does DSM-BIA improve accuracy compared to whole-body BIA?
A. It measures each segment (RA, LA, TR, RL, LL) individually, reducing trunk error.

Q3. Does InBody use empirical prediction equations?
A. No, InBody calculates composition directly from impedance, reactance, and resistance.

Q4. What frequencies are typically used in InBody devices?
A. Multi-frequency range from 1 kHz to 3000 kHz.

Q5. Why is reactance clinically relevant?
A. It reflects cell membrane integrity and correlates with phase angle.

## 2. Hydration, Fluids & Critical Care

Q6. What is the normal ECW/TBW range?
A. 0.360–0.390.

Q7. What clinical states elevate ECW/TBW?
A. Edema, heart failure, renal dysfunction, inflammation, sepsis.

Q8. Can InBody detect dehydration?
A. Yes, via increased resistance and reduced ICW/ECW.

Q9. How does InBody differentiate muscle loss vs dehydration?
A. Muscle loss → ICW decreases disproportionately; dehydration → proportional ICW & ECW decrease.

Q10. Is InBody safe for ICU patients?
A. Yes, especially with S10 supine electrodes.

Q11. How does fluid overload present on InBody?
A. Elevated ECW, high ECW/TBW, segmental ECW elevation.

Q12. Can InBody track sepsis recovery?
A. Yes; improving ECW/TBW indicates resolving inflammation.

Q13. What is the most reliable hydration status marker on InBody?
A. ECW/TBW ratio.

## 3. Muscle Mass & Sarcopenia

Q14. What does SMM include?
A. Only contractile skeletal muscle tissue.

Q15. What parameter is used for sarcopenia diagnosis?
A. SMI , HGS

Q16. What are the sarcopenia cut-offs (Asian)?
A. Men < 7.0 kg/m², Women < 5.7 kg/m².

Q17. Best marker of muscle quality?
A. ICW or ICW/ECW ratio along with SMM levels.

Q18. What causes discrepancies in segmental lean mass?
A. Injury, extensive dominant limb usage , post-surgical changes, nutritional deficits.

Q19. What trend indicates sarcopenic obesity?
A. Low SMM + high visceral fat.

Q20. How often should muscle be monitored in elderly?
A. Every 4–6 weeks.

## 4. Fat Mass, VAT & Metabolic Health

Q21. What is Visceral Fat Level (VFL)?
A. An index reflecting visceral adiposity.

Q22. What range indicates high metabolic risk?
A. VFL ≥ 10.

Q23. Does InBody directly measure visceral fat area?
A. It provides a validated estimated index correlated to imaging trends.

Q24. Can InBody distinguish subcutaneous vs visceral fat?
A. Yes, through trunk fat and visceral fat index estimates.

Q25. What changes reflect improving insulin sensitivity?
A. ↓ VFL, ↓ WHR, ↑ SMM.

Q26. Is BAT/WAT measurable by InBody?
A. No, but related metabolic parameters can be monitored.

Q27. What parameter correlates most with NAFLD risk?
A. High visceral fat + high trunk fat.

## 5. Endocrine & Metabolic Disorders

Q28. How is InBody useful in PCOS?
A. Tracks visceral fat and lean mass changes.

Q29. What patterns indicate metabolic syndrome?
A. High VFL, high WHR, low SMM.

Q30. What reflects improved metabolic health on follow-up?
A. Reduced fat mass with stable or rising SMM.

Q31. Can hypothyroidism affect InBody readings?
A. Yes—slower metabolism → fat gain, mild fluid retention.

Q32. Which parameter changes earliest in insulin resistance?
A. Visceral fat & Lower limb lean mass.

## 6. Bariatric Surgery & Obesity Medicine

Q33. What pre-bariatric markers are essential?
A. SMM, ALM, ECW/TBW, visceral fat, PBF , Phase Angle(>3.9).

Q34. What is the typical early post-op pattern?
A. Rapid fat loss, moderate initial SMM loss.

Q35. Why monitor ICW post bariatric surgery?
A. ICW reflects muscle recovery and nutrient status. Helps monitor weight recycling

Q36. Ideal testing frequency post-surgery?
A. Every 2–4 weeks.

## 7. Renal, Hepatic & Medical Conditions

Q37. How is InBody used in CKD?
A. ECW/TBW guides fluid management and sarcopenia diagnosis.

Q38. How does dialysis affect readings?
A. Post-dialysis ECW decreases significantly.

Q39. Can InBody detect ascites?
A. Indirectly via elevated trunk ECW.

Q40. What indicates hepatic inflammation?
A. Elevated ECW/TBW and reduced phase angle.

Q41. What pattern is common in NAFLD?
A. High visceral fat + increased trunk fat.

## 8. Sports Medicine & Rehabilitation

Q42. Can InBody detect muscle asymmetry?
A. Yes, via segmental lean analysis.

Q43. What indicates overtraining?
A. Reduced SMM or ICW despite training load.

Q44. How is InBody useful in Anterior Cruciate Ligament (ACL) ?
A. Tracks limb muscle recovery and balance.

Q45. What parameter reflects training adaptation?
A. Increased ICW and segmental lean mass.

Q46. What indicates poor recovery?
A. High ECW in the affected limb/ body part .

## 9. Testing Protocol & Technical Considerations

Q47. Optimal pre-test conditions?
A. 2–3 hours post-meal, empty bladder, no exercise 6–12 hours.

Q48. Can menstruation influence results?
A. Yes, ECW may increase.

Q49. Is footwear allowed during testing?
A. No, bare feet required for electrode contact.

Q50. Can metal implants affect results?
A. Usually minimal effect; readings remain valid.

Q51. What causes sudden fat mass spikes?
A. Fluid retention, menstrual cycle, hydration changes.

Q52. Why compare results only on the same device model?
A. Different models may use different frequency combinations hence results may vary.